Εθνικό Μετσόβιο Πολυτεχνείο

Σχολή Ηλεκτρολόγων Μηχανικών και Μηχανικών

Υπολογιστών

Έτος: 2024- 2025

Ψηφιακά VLSI

Ομάδα 45

Παπαδόπουλος Χαράλαμπος (ΑΜ: 03120199)

Νικόλαος Παπακωνσταντόπουλος (ΑΜ: 03120069)

# Υλοποίηση Φίλτρου FIR

Στη γενική περίπτωση, η σχέση εισόδου - εξόδου ενός FIR φίλτρου είναι η ακόλουθη:

Όπου

- M είναι η τάξη του φίλτρου

- y[n] είναι η έξοδος του φίλτρου τη διακριτή χρονική στιγμή n

- h[k] είναι ο k-οστός συντελεστής του φίλτρου, με k = 0, 1, 2, ..., M

- x[n] είναι η τιμή του σήματος εισόδου τη διακριτή χρονική στιγμή n

Στην συγκεκριμένη άσκηση μας ζητήθηκε η υλοποίηση ενός 8-tap FIR για N=8 bits εύρος δεδομένων Χ σύμφωνα με την ακόλουθη αρχιτεκτονική.

Ακολουθούν οι κώδικες μαζί με τα RTL της κάθε δομικής μονάδας.

ROM

library IEEE; use IEEE.STD_LOGIC_1164.ALL; use ieee.std_logic_unsigned.all; entity rom is generic ( coeff_width : integer :=8 --- width of coefficients (bits) ); Port ( clk : in STD_LOGIC; en : in STD_LOGIC; --- operation enable addr : in STD_LOGIC_VECTOR (2 downto 0); -- memory address rom_out : out STD_LOGIC_VECTOR (coeff_width-1 downto 0)); -- output data end rom; architecture Behavioral of rom is type rom_type is array (7 downto 0) of std_logic_vector (coeff_width-1 downto 0); signal rom : rom_type:= ("00001000", "00000111", "00000110", "00000101", "00000100", "00000011", "00000010", "00000001"); -- initialization of rom with user data signal rdata : std_logic_vector(coeff_width-1 downto 0) := (others => '0'); begin rdata <= rom(conv_integer(addr)); process (clk) begin if (clk'event and clk = '1') then if (en = '1') then rom_out <= rdata; end if; end if; end process; end Behavioral;

RAM

library ieee; use ieee.std_logic_1164.all; use ieee.std_logic_unsigned.all; entity ram is generic ( data_width : integer :=8 --- width of data (bits) ); port (clk : in std_logic; rst : in std_logic; we : in std_logic; --- memory write enable en : in std_logic; --- operation enable addr : in std_logic_vector(2 downto 0); -- memory address di : in std_logic_vector(data_width-1 downto 0); -- input data do : out std_logic_vector(data_width-1 downto 0)); -- output data end ram; architecture Behavioral of ram is type ram_type is array (7 downto 0) of std_logic_vector (data_width-1 downto 0); signal RAM : ram_type := (others => (others => '0')); -- initialize at 0 begin process (clk, rst) begin if rst = '1' then RAM <= (others => (others => '0')); -- reset to 0 else if clk'event and clk = '1' then if en = '1' then if we = '1' then -- write operation RAM(7 downto 1) <= RAM(6 downto 0); RAM(conv_integer(addr)) <= di; do <= di; else -- read operation do <= RAM(conv_integer(addr)); end if; end if; end if; end if; end process; end Behavioral;

Περικομμένο RTL. Κανονικά έχει κι άλλα (πανομοιότυπα) στάδια

MAC

library ieee; use ieee.std_logic_1164.all; use ieee.std_logic_unsigned.all; entity mac is Port ( clk : in std_logic; mac_init : in std_logic; rom_out : in std_logic_vector(8-1 downto 0); ram_out : in std_logic_vector(8-1 downto 0); y : out std_logic_vector(19-1 downto 0) ); end mac; architecture behavioral of mac is signal accumulator : std_logic_vector(19-1 downto 0) := (others => '0'); begin process(clk) begin if rising_edge(clk) then if mac_init = '1' then accumulator <= "000" & (ram_out * rom_out); else accumulator <= accumulator + (ram_out * rom_out); end if; end if; end process; y <= accumulator; end behavioral;

Control Unit

library IEEE; use IEEE.std_logic_1164.ALL; use IEEE.std_logic_unsigned.ALL; use ieee.numeric_std.all; entity control_unit is Port ( clk : in std_logic; valid_in : in std_logic; rom_address : out std_logic_vector(3-1 downto 0); ram_address : out std_logic_vector(3-1 downto 0); mac_init : out std_logic; we : out std_logic; -- write enable for ram valid_out : out std_logic; temp_counter : out std_logic_vector(2 downto 0) ); end control_unit; architecture Behavioral of control_unit is signal counter : std_logic_vector(2 downto 0):= (others => '0'); signal temp_valid_out : std_logic_vector(9-1 downto 0) := (others => '0'); begin process(clk) begin if rising_edge(clk) then if counter = "000" then we <= valid_in; mac_init <= '1'; temp_valid_out(0) <= valid_in; else temp_valid_out(0) <= valid_in; mac_init <= '0'; we <= '0'; end if; rom_address <= counter; ram_address <= counter; -- increment the counter only if valid_in is high so we write a new value to the ram -- or if valid_in is low and the counter is not zero (so we can read the ram) if not (valid_in = '0' and counter = "000" ) then counter <= std_logic_vector(unsigned(counter) + 1); end if; valid_out <= temp_valid_out(8); end if; end process; process(clk) begin if rising_edge(clk) then temp_valid_out(8 downto 1) <= temp_valid_out(7 downto 0); end if; end process; process(counter) begin temp_counter <= counter; end process; end Behavioral;

FIR

library IEEE; use IEEE.std_logic_1164.ALL; use IEEE.numeric_std.all; entity fir is Port ( valid_in : in std_logic; clk : in std_logic; rst : in std_logic; x : in std_logic_vector(8-1 downto 0); valid_out : out std_logic; y : out std_logic_vector(19-1 downto 0); -- Debug ports dbg_counter : out std_logic_vector(2 downto 0); dbg_rom_out : out std_logic_vector(7 downto 0); dbg_ram_out : out std_logic_vector(7 downto 0); dbg_mac_init : out std_logic; dbg_we : out std_logic; -- Additional debug ports dbg_x_register : out std_logic_vector(7 downto 0); dbg_ram_di : out std_logic_vector(7 downto 0); dbg_ram_addr : out std_logic_vector(2 downto 0); dbg_mac_acc : out std_logic_vector(18 downto 0) ); end fir; architecture structural of fir is component mac is Port ( clk : in std_logic; mac_init : in std_logic; rom_out : in std_logic_vector(8-1 downto 0); ram_out : in std_logic_vector(8-1 downto 0); y : out std_logic_vector(19-1 downto 0) ); end component; component rom is generic ( coeff_width : integer :=8 --- width of coefficients (bits) ); Port ( clk : in STD_LOGIC; en : in STD_LOGIC; --- operation enable addr : in STD_LOGIC_VECTOR (2 downto 0); -- memory address rom_out : out STD_LOGIC_VECTOR (coeff_width-1 downto 0)); -- output data end component; component ram is generic ( data_width : integer :=8 --- width of data (bits) ); port (clk : in std_logic; rst : in std_logic; we : in std_logic; --- memory write enable en : in std_logic; --- operation enable addr : in std_logic_vector(2 downto 0); -- memory address di : in std_logic_vector(data_width-1 downto 0); -- input data do : out std_logic_vector(data_width-1 downto 0)); -- output data end component; component control_unit is Port ( clk : in std_logic; valid_in : in std_logic; rom_address : out std_logic_vector(3-1 downto 0); ram_address : out std_logic_vector(3-1 downto 0); mac_init : out std_logic; we : out std_logic; -- write enable for ram valid_out : out std_logic ); end component; signal rom_out_int : std_logic_vector(8-1 downto 0); signal ram_out_int : std_logic_vector(8-1 downto 0); signal mac_init : std_logic; signal we : std_logic; signal rom_address : std_logic_vector(3-1 downto 0); signal ram_address : std_logic_vector(3-1 downto 0); signal mac_init_register : std_logic; signal x_register : std_logic_vector(8-1 downto 0); -- Add new internal signal for MAC output signal mac_output : std_logic_vector(19-1 downto 0); begin -- stage 1: control unit and register for x control_unit_inst : control_unit port map ( clk => clk, valid_in => valid_in, rom_address => rom_address, ram_address => ram_address, mac_init => mac_init, we => we, valid_out => valid_out ); x_reg_proc : process(clk, rst) begin if rst = '1' then x_register <= (others => '0'); elsif rising_edge(clk) then x_register <= x; end if; end process; -- stage 2: rom and ram and mac_init register rom_inst : rom port map ( clk => clk, en => '1', addr => rom_address, rom_out => rom_out_int ); ram_inst : ram port map ( clk => clk, rst => rst, we => we, en => '1', addr => ram_address, di => x_register, do => ram_out_int ); mac_init_register_proc : process(clk, rst) begin if rst = '1' then mac_init_register <= '0'; elsif rising_edge(clk) then mac_init_register <= mac_init; end if; end process; -- stage 3: mac mac_inst : mac port map ( clk => clk, mac_init => mac_init_register, rom_out => rom_out_int, ram_out => ram_out_int, y => mac_output -- Connect to internal signal ); -- Connect output port y <= mac_output; -- Debug assignments dbg_counter <= rom_address; dbg_rom_out <= rom_out_int; dbg_ram_out <= ram_out_int; dbg_mac_init <= mac_init; dbg_we <= we; -- Add new debug assignments dbg_x_register <= x_register; dbg_ram_di <= x_register; -- RAM data input dbg_ram_addr <= ram_address; dbg_mac_acc <= mac_output; -- Use internal signal instead of y end structural;

FIR Testbench

library IEEE; use IEEE.STD_LOGIC_1164.ALL; use IEEE.STD_LOGIC_UNSIGNED.ALL; use IEEE.NUMERIC_STD.ALL; use IEEE.math_real.all; entity fir_tb is end fir_tb; architecture testbench of fir_tb is constant data_width, coeff_width : integer := 8; constant min_vector_size : integer := 8; component fir is Port ( valid_in : in std_logic; clk : in std_logic; rst : in std_logic; x : in std_logic_vector(8-1 downto 0); valid_out : out std_logic; y : out std_logic_vector(19-1 downto 0); -- Debug ports dbg_counter : out std_logic_vector(2 downto 0); dbg_rom_out : out std_logic_vector(7 downto 0); dbg_ram_out : out std_logic_vector(7 downto 0); dbg_mac_init : out std_logic; dbg_we : out std_logic; dbg_x_register : out std_logic_vector(7 downto 0); dbg_ram_di : out std_logic_vector(7 downto 0); dbg_ram_addr : out std_logic_vector(2 downto 0); dbg_mac_acc : out std_logic_vector(18 downto 0) ); end component; --Input Signals signal clk: std_logic:='0'; signal rst: std_logic:='0'; signal valid_in: std_logic:='0'; signal x : std_logic_vector(8-1 downto 0):=(others=>'0'); signal valid_out: std_logic:='0'; --Output Signals signal y : std_logic_vector(19-1 downto 0):=(others=>'0'); -- Debug Signals signal dbg_counter : std_logic_vector(2 downto 0); signal dbg_rom_out : std_logic_vector(7 downto 0); signal dbg_ram_out : std_logic_vector(7 downto 0); signal dbg_mac_init : std_logic; signal dbg_we : std_logic; signal dbg_x_register : std_logic_vector(7 downto 0); signal dbg_ram_di : std_logic_vector(7 downto 0); signal dbg_ram_addr : std_logic_vector(2 downto 0); signal dbg_mac_acc : std_logic_vector(18 downto 0); --Clock constant CLK_PERIOD : time := 10ns; begin clk_process: process begin clk <= '0'; wait for CLK_PERIOD/2; clk <= '1'; wait for CLK_PERIOD/2; end process; UUT: fir port map ( clk => clk, rst => rst, valid_in => valid_in, x => x , y => y, valid_out => valid_out, -- Debug connections dbg_counter => dbg_counter, dbg_rom_out => dbg_rom_out, dbg_ram_out => dbg_ram_out, dbg_mac_init => dbg_mac_init, dbg_we => dbg_we, dbg_x_register => dbg_x_register, dbg_ram_di => dbg_ram_di, dbg_ram_addr => dbg_ram_addr, dbg_mac_acc => dbg_mac_acc ); TEST: process begin rst <= '1'; valid_in <= '1'; x <= x"01"; wait for CLK_PERIOD/4; rst <= '0'; wait for 3*CLK_PERIOD/4; valid_in <= '0'; wait for 7*CLK_PERIOD; valid_in <= '1'; x <= x"02"; wait for CLK_PERIOD; valid_in <= '0'; wait for 7*CLK_PERIOD; valid_in <= '1'; x <= x"03"; wait for CLK_PERIOD; valid_in <= '0'; wait for 7*CLK_PERIOD; valid_in <= '1'; x <= x"04"; wait for CLK_PERIOD; valid_in <= '0'; wait for 7*CLK_PERIOD; valid_in <= '1'; x <= x"05"; wait for CLK_PERIOD; valid_in <= '0'; wait for 7*CLK_PERIOD; valid_in <= '1'; x <= x"06"; wait for CLK_PERIOD; valid_in <= '0'; wait for 7*CLK_PERIOD; valid_in <= '1'; x <= x"07"; wait for CLK_PERIOD; valid_in <= '0'; wait for 7*CLK_PERIOD; valid_in <= '1'; x <= x"08"; wait for CLK_PERIOD; valid_in <= '0'; wait for 7*CLK_PERIOD; valid_in <= '1'; x <= x"09"; wait for CLK_PERIOD; valid_in <= '0'; wait for 15*CLK_PERIOD; -- new input arrives delayed valid_in <= '1'; x <= x"10"; wait for CLK_PERIOD; valid_in <= '0'; wait for 10*CLK_PERIOD; -- new invalid input arrives valid_in <= '1'; x <= x"10"; wait for CLK_PERIOD; valid_in <= '0'; wait for 10*CLK_PERIOD; rst <= '1'; valid_in <= '1'; x <= x"01"; wait for CLK_PERIOD/4; rst <= '0'; wait for 3*CLK_PERIOD/4; valid_in <= '0'; rst <= '1'; valid_in <= '1'; x <= x"ff"; wait for CLK_PERIOD/4; rst <= '0'; wait for 3*CLK_PERIOD/4; valid_in <= '0'; wait for 7*CLK_PERIOD; valid_in <= '1'; x <= x"ff"; wait for CLK_PERIOD; valid_in <= '0'; wait for 7*CLK_PERIOD; valid_in <= '1'; x <= x"ff"; wait for CLK_PERIOD; valid_in <= '0'; wait for 7*CLK_PERIOD; valid_in <= '1'; x <= x"ff"; wait for CLK_PERIOD; valid_in <= '0'; wait for 7*CLK_PERIOD; valid_in <= '1'; x <= x"ff"; wait for CLK_PERIOD; valid_in <= '0'; wait for 7*CLK_PERIOD; valid_in <= '1'; x <= x"ff"; wait for CLK_PERIOD; valid_in <= '0'; wait for 7*CLK_PERIOD; valid_in <= '1'; x <= x"ff"; wait for CLK_PERIOD; valid_in <= '0'; wait for 7*CLK_PERIOD; valid_in <= '1'; x <= x"ff"; wait for CLK_PERIOD; valid_in <= '0'; wait for 7*CLK_PERIOD; valid_in <= '1'; x <= x"ff"; wait for CLK_PERIOD; valid_in <= '0'; wait for 7*CLK_PERIOD; wait; end process; end testbench;

synthesis Critical Path: rom→mac=6.216ns

Επειδή το critical path από το implementation είναι μεγαλύτερο (7.989 ns) θα λάβουμε αυτό υπόψη για να βρούμε τη μέγιστη συχνότητα στην οποία μπορεί να λειτουργήσει το FPGA:

fFPGA≤1critical path time≃ 125 MHz (στρογγυλοποίηση προς τα κάτω)

Και οι πόροι που χρησιμοποιήθηκαν (Post Implementation Utilization):